Practical No. 3

- Using Packet Tracer, create a basic network of One server and two computers using appropriate network wire. Use Dynamic IP address allocation and show connectivity.

Step 1: Select ‘End Devices’ and select ‘Server’

Step 2: Drag and drop the ‘Server’

Step 3: Select ‘Network Devices’, select ‘Switch’ and  select ‘PT Switch’

Step 4: Drag and drop the ‘PT Switch’

Step 5: Select ‘End Devices’ and select ‘PC’ and ‘Laptop’

Step 6: Drag and drop 2 ‘PC’s

Step 7: Select ‘Connections’ and select ‘Copper Straight Through’

Step 8: Connect all devices with fastethernet

Step 9: Give start IP name 192.168.0.0

Step 10: Select ‘Server’ and go to ‘IP Configuration’

Step 11: Go to ‘Desktop’ give ‘Static IP’ and  ‘subnet nask’

Step 12: Go to ‘Services’ and give default gateaway and DNS

Step 13: Click ‘save’

Step 14: Select every PC and select DHCP

Step 15: Connections Completed

Step 16: Assign Packets

Step 17: Run Simulation